- Setting Arduino

- Setting Preferences (File – Preferences)

- Setting Board (Tools – Board – ESP8266 Boards – NodeMCU 1.0)

- Setting Upload Speed (Tools – Upload Speed – 115200)

- Setting Flash Size (Tools – Flash Size – 4MB FS:3MB OTA:~512KB)

- Setting Libraries (Tools – Manage Libraries…)

- Install Library Board of Adafruit ESP8266

- Install Library of DHT sensor library

- Sensor Temperature LM35

- Sensor Humidity DHT11

#include "DHT.h"
#define DHTPIN D1
#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

float vref = 3.3;
float res = vref/1023;

void setup(){
  Serial.begin(9600);
  dht.begin();
}
void loop()
{ 
  Serial.println(" ====== DHT11 SENSOR ====== ");
  
  float hum = dht.readHumidity();
  float tem = dht.readTemperature();
  Serial.print(" Humidity DHT11: ");
  Serial.print(hum);
  Serial.print("% ");
  Serial.print("\n");
  Serial.print(" Temperature DHT11: ");
  Serial.print(tem);
  Serial.print("°C ");
  Serial.print(">>> ");
  Serial.print((int)round(1.8*tem+32));
  Serial.println("°F ");

  Serial.println(" ====== LM35 SENSOR ====== ");

   float temp = analogRead(A0); 
   temp = (temp*res); 
   temp = temp*100; 
   Serial.print(" Temperature LM35: ");
   Serial.print(temp);

Serial.println("°C ");

Serial.println(" ========================== ");

delay(10000); //waiting for 10 seconds

}

BLYNK

#define BLYNK_PRINT Serial

#include <BlynkSimpleEsp8266.h>

#include <SPI.h>

#include <ESP8266WiFi.h>

#include "DHT.h"

char auth[] = "S6kgpOckbJWevYez5Rxrr2kKcLAw5SXj";

char ssid[] = "Azzam";

char pass[] = "azzam53a";

#define DHTPIN D1

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

SimpleTimer timer;

float vref = 3.3;

float res = vref/1023;

void sendSensor()

{

float hum = dht.readHumidity();

float tem = dht.readTemperature(); // or dht.readTemperature(true) for Fahrenheit

if (isnan(hum) || isnan(tem)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// You can send any value at any time.

// Please don't send more that 10 values per second.

Blynk.virtualWrite(V5, tem);

Blynk.virtualWrite(V6, hum);

}

void setup(){

Serial.begin(9600);

Blynk.begin(auth, ssid, pass);

dht.begin();

timer.setInterval(1000L, sendSensor);

}

void loop()

{

Serial.println(" ====== DHT11 SENSOR ====== ");

float hum = dht.readHumidity();

float tem = dht.readTemperature();

Serial.print(" Humidity DHT11: ");

Serial.print(hum);

Serial.print("% ");

Serial.print("\n");

Serial.print(" Temperature DHT11: ");

Serial.print(tem);

Serial.print("°C ");

Serial.print(">>> ");

Serial.print((int)round(1.8*tem+32));

Serial.println("°F ");

Serial.println(" ====== LM35 SENSOR ====== ");

float temp = analogRead(A0);

temp = (temp*res);

temp = temp*100;

Serial.print(" Temperature LM35: ");

Serial.print(temp);

Serial.println("°C ");

Serial.println(" ========================== ");

Blynk.run();

timer.run();

delay(10000); //waiting for 10 seconds

}

Reference:

Power Point - E2. Microcontroller_NodeMCU

https://medium.com/@andreanewgate/menggunakan-sensor-suhu-dan-kelembapan-dht11-dengan-nodemcu-b7976f527583

https://forum.arduino.cc/index.php?topic=171299.0

http://kursuselektronikaku.blogspot.com/2019/10/iot-project-blynk-nodemcu-esp8266.html

https://kelasrobot.com/solved-cara-mengatasi-error-login-timeout-blynk/